DOIP Vehicle Identification and Diagnostic Communication Note

1. DOIP Overview

DOIP (Diagnostics over IP) is part of the UDS protocol stack and is defined in ISO 13400. It allows diagnostic tools to communicate with vehicle ECUs over Ethernet (TCP/IP).

2. Vehicle Identification Process

Step 1: Socket Connection

- Establish a TCP connection to the vehicle’s DOIP gateway.

- Default port: 13400 (per ISO 13400-2)

Step 2: Send Vehicle Identification Request

- Format: DOIP Vehicle Identification Request Message

- Message Type: 0x0002 (Vehicle Identification Request)

- Payload: optional VIN request using logical address

Example Request Message:

02 FD 00 02 00 00 00 00

(Where 02 FD is the protocol version and inverse, 00 02 is message type)

Step 3: Receive Vehicle Identification Response

- Message Type: 0x0004 (Vehicle Identification Response)

- Payload contains:

  - VIN (Vehicle Identification Number)

  - Logical Address of the ECU

  - Additional information if available (OEM specific)

3. Send UDS Diagnostic Request

Step 1: Routing Activation

- Message Type: 0x0005 (Routing Activation Request)

- Required to activate a communication path between tester and ECU.

Step 2: Send Diagnostic Message

- Message Type: 0x8001 (Diagnostic Message)

- UDS Service (e.g., Read DTCs, ECU Reset, etc.)

- Use the logical address obtained during vehicle identification.

Example: Read ECU Identification

hex

CopyEdit

02 FD 80 01 [Logical Address] 22 F1 90

Where 22 F1 90 is a UDS request to read the VIN.

4. Receive UDS Response

- Message Type: 0x8002 (Diagnostic Message Response)

- Response will contain the data from the ECU (e.g., VIN, ECU name, DTCs)

📝 Summary Flow

- Connect to port 13400 (TCP).

- Send Vehicle Identification Request (0x0002).

- Receive Vehicle Identification Response (0x0004).

- Perform Routing Activation (0x0005).

- Send UDS diagnostic request (0x8001).

- Receive response (0x8002).

Let me know if you’d like example code (e.g., Python socket communication) or specific message structure details.

You said:

metion agin with frame

ChatGPT said:

Certainly! Here's the DOIP Vehicle Identification and UDS Communication Process explained again with frame structures for each step — including headers, message types, and payload formats.

🚗 DOIP Diagnostic Communication with Frames

1. TCP Connection

- Port: 13400

- Protocol: TCP/IP (ISO 13400-2)

2. Vehicle Identification Request

📤 Request Frame

| Field | Value | Description |
| --- | --- | --- |
| Protocol Version | 0x02 | DOIP version |
| Inverse Version | 0xFD | 0xFF - version |
| Payload Type | 0x0002 | Vehicle Identification Request |
| Payload Length | 0x00000000 | No additional data |

Hex Frame:

CopyEdit

02 FD 00 02 00 00 00 00

3. Vehicle Identification Response

📥 Response Frame

| Field | Example Value | Description |
| --- | --- | --- |
| Protocol Version | 0x02 | DOIP version |
| Inverse Version | 0xFD | Inverse |
| Payload Type | 0x0004 | Vehicle Identification Response |
| Payload Length | 0x00000015 | e.g., 21 bytes |
| VIN (17 bytes) | e.g. 4A4MM21S56E000001 | ASCII-encoded VIN |
| Logical Address | 0x0E00 | ECU logical address |

Example Frame (Simplified):

mathematica

CopyEdit

02 FD 00 04 00 00 00 15 34 41 34 4D 4D 32 31 53 35 36 45 30 30 30 30 30 31 0E 00

4. Routing Activation Request

📤 Request Frame

| Field | Value | Description |
| --- | --- | --- |
| Protocol Version | 0x02 | DOIP version |
| Inverse Version | 0xFD | Inverse |
| Payload Type | 0x0005 | Routing Activation Request |
| Payload Length | 0x0000000C | 12 bytes |
| Source Address | 0x0E00 | Tester address |
| Activation Type | 0x0000 | Default |
| Reserved/Optional | 8 bytes | Reserved/optional fields |

Example Frame (Simplified):

mathematica

CopyEdit

02 FD 00 05 00 00 00 0C 0E 00 00 00 00 00 00 00 00 00 00

5. Diagnostic Message (UDS Request)

📤 Request Frame

| Field | Value | Description |
| --- | --- | --- |
| Protocol Version | 0x02 | DOIP version |
| Inverse Version | 0xFD | Inverse |
| Payload Type | 0x8001 | Diagnostic Message |
| Payload Length | Varies | e.g. 8 bytes |
| Target Address | 0x0E00 | ECU logical address |
| UDS Service | e.g. 22 F1 90 | Read VIN from ECU |

Example Frame (Read VIN):

mathematica

CopyEdit

02 FD 80 01 00 00 00 05 0E 00 22 F1 90

6. Diagnostic Message Response

📥 Response Frame

| Field | Value | Description |
| --- | --- | --- |
| Protocol Version | 0x02 | DOIP version |
| Inverse Version | 0xFD | Inverse |
| Payload Type | 0x8002 | Diagnostic Message Response |
| Payload Length | Varies | Based on UDS response |
| Source Address | 0x0E00 | ECU logical address |
| UDS Response | e.g. 62 F1 90 <VIN> | VIN data response |

Example Frame (Simplified):

02 FD 80 02 00 00 00 14 0E 00 62 F1 90 4A 4A 32 31 58 ...

📝 Sequence Summary (with Frame Types)

| Step | Message Type | Description |
| --- | --- | --- |
| 1 | 0x0002 | Vehicle Identification Request |
| 2 | 0x0004 | Vehicle Identification Response |
| 3 | 0x0005 | Routing Activation Request |
| 4 | 0x8001 | Send UDS Request |
| 5 | 0x8002 | Receive UDS Response |

# DHCP Client (Discover → Offer → Request → ACK)

cpp

CopyEdit

// dhcp_client.cpp

#include <bits/stdc++.h>

#include <unistd.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <arpa/inet.h>

#define DHCP_SERVER_PORT 67

#define DHCP_CLIENT_PORT 68

#define BOOTREQUEST 1

#define BOOTREPLY   2

#define HTYPE_ETH   1

#define HLEN_ETH    6

#define DISCOVER    1

#define OFFER       2

#define REQUEST     3

#define ACK         5

#define OPT_MSG_TYPE    53

#define OPT_REQ_IP      50

#define OPT_SERVER_ID   54

#define OPT_END         255

#pragma pack(push,1)

struct DhcpPacket {

uint8_t op, htype, hlen, hops;

uint32_t xid;

uint16_t secs, flags;

uint32_t ciaddr, yiaddr, siaddr, giaddr;

uint8_t chaddr[16];

uint8_t sname[64], file[128], options[312];

};

#pragma pack(pop)

void add_option(uint8_t *opt, int &idx, uint8_t code, uint8_t len, const void *val) {

opt[idx++] = code;

opt[idx++] = len;

memcpy(opt+idx, val, len);

idx += len;

}

int main() {

int sock = socket(AF_INET, SOCK_DGRAM, IPPROTO_UDP);

int on = 1; setsockopt(sock, SOL_SOCKET, SO_BROADCAST, &on, sizeof(on));

sockaddr_in cli = {AF_INET, htons(DHCP_CLIENT_PORT), INADDR_ANY};

bind(sock, (sockaddr*)&cli, sizeof(cli));

DhcpPacket pkt{};

srand(time(nullptr));

pkt.op = BOOTREQUEST; pkt.htype=HTYPE_ETH; pkt.hlen=HLEN_ETH;

pkt.xid = htonl(rand()); pkt.flags = htons(0x8000);

uint8_t mac[6]={0xaa,0xbb,0xcc,0xdd,0xee,0xff};

memcpy(pkt.chaddr, mac, 6);

uint8_t idx=0;

uint32_t cookie = htonl(0x63825363);

memcpy(pkt.options, &cookie, 4); idx=4;

add_option(pkt.options, idx, OPT_MSG_TYPE,1,(uint8_t[]){DISCOVER});

pkt.options[idx++] = OPT_END;

sockaddr_in dest={AF_INET,htons(DHCP_SERVER_PORT),INADDR_BROADCAST};

sendto(sock, &pkt, sizeof(pkt),0,(sockaddr*)&dest,sizeof(dest));

std::cout<<"DHCP Discover sent.\n";

while(true) {

DhcpPacket rep{};

socklen_t len=sizeof(dest);

recvfrom(sock,&rep,sizeof(rep),0,(sockaddr*)&dest,&len);

if(rep.op!=BOOTREPLY) continue;

uint8_t msgType=0; uint32_t serverId=0;

uint8_t *opt=rep.options;

for(int i=4; i<312;) {

uint8_t c=opt[i++]; if(c==OPT_END) break;

uint8_t l=opt[i++];

if(c==OPT_MSG_TYPE) msgType=opt[i];

else if(c==OPT_SERVER_ID) memcpy(&serverId,opt+i,4);

i+=l;

}

if(msgType==OFFER) {

std::cout<<"Got Offer: "<<inet_ntoa(*(in_addr*)&rep.yiaddr)<<"\n";

DhcpPacket req = rep;

req.op=BOOTREQUEST; req.xid = rep.xid;

idx=0; memcpy(req.options,&cookie,4); idx=4;

add_option(req.options,idx,OPT_MSG_TYPE,1,(uint8_t[]){REQUEST});

add_option(req.options,idx,OPT_REQ_IP,4,&rep.yiaddr);

add_option(req.options,idx,OPT_SERVER_ID,4,&serverId);

req.options[idx++]=OPT_END;

sendto(sock,&req,sizeof(req),0,(sockaddr*)&dest,sizeof(dest));

std::cout<<"DHCP Request sent.\n";

} else if(msgType==ACK) {

std::cout<<"Received ACK: IP leased "<<inet_ntoa(*(in_addr*)&rep.yiaddr)<<"\n";

break;

}

}

close(sock);

return 0;

}

🛠 DHCP Server (Offer → ACK)

cpp

CopyEdit

// dhcp_server.cpp

#include <bits/stdc++.h>

#include <unistd.h>

#include <sys/socket.h>

#include <netinet/in.h>

#include <arpa/inet.h>

#define CLIENT_PORT 68

#define SERVER_PORT 67

#define BOOTREQUEST 1

#define BOOTREPLY   2

#define OFFER       2

#define ACK         5

#define OPT_MSG_TYPE 53

#define OPT_SERVER_ID 54

#define OPT_END        255

#pragma pack(push,1)

struct DhcpPacket { /* same as client struct */ uint8_t data[0]; };

#pragma pack(pop)

uint8_t* find_opt(uint8_t *opt, int code) {

for(int i=4;i<312;) {

uint8_t c=opt[i++]; if(c==OPT_END) break;

uint8_t l=opt[i++];

if(c==code) return &opt[i];

i += l;

}

return nullptr;

}

int main(){

int rawsock = socket(AF_INET, SOCK_DGRAM, IPPROTO_UDP);

int on=1; setsockopt(rawsock, SOL_SOCKET, SO_REUSEADDR, &on, sizeof(on));

sockaddr_in srv{AF_INET,htons(SERVER_PORT),INADDR_ANY};

bind(rawsock,(sockaddr*)&srv,sizeof(srv));

std::cout<<"DHCP Server listening...\n";

while(true){

DhcpPacket pkt{};

sockaddr_in cli; socklen_t len=sizeof(cli);

recvfrom(rawsock,&pkt,sizeof(pkt),0,(sockaddr*)&cli,&len);

if(pkt.op!=BOOTREQUEST) continue;

uint8_t *mt = find_opt(pkt.options, OPT_MSG_TYPE);

if(!mt) continue;

uint8_t type = *mt;

if(type==1){ // DISCOVER

pkt.op=BOOTREPLY;

pkt.siaddr = inet_addr("192.168.1.1");

pkt.yiaddr = inet_addr("192.168.1.100");

uint8_t idx=0;

uint32_t cookie=htonl(0x63825363);

memcpy(pkt.options,&cookie,4); idx=4;

pkt.options[idx++]=OPT_MSG_TYPE; pkt.options[idx++]=1; pkt.options[idx++]=OFFER;

pkt.options[idx++]=OPT_SERVER_ID; pkt.options[idx++]=4;

uint32_t sid = srv.sin_addr.s_addr;

memcpy(pkt.options+idx,&sid,4); idx+=4;

pkt.options[idx++]=OPT_END;

sockaddr_in dest{AF_INET,htons(CLIENT_PORT),INADDR_BROADCAST};

sendto(rawsock,&pkt,sizeof(pkt),0,(sockaddr*)&dest,sizeof(dest));

std::cout<<"Sent OFFER to "<<inet_ntoa(cli.sin_addr)<<"\n";

}

else if(type==3){ // REQUEST

pkt.op=BOOTREPLY;

pkt.siaddr = inet_addr("192.168.1.1");

uint8_t idx=0;

uint32_t cookie=htonl(0x63825363);

memcpy(pkt.options,&cookie,4); idx=4;

pkt.options[idx++]=OPT_MSG_TYPE; pkt.options[idx++]=1; pkt.options[idx++]=ACK;

pkt.options[idx++]=OPT_SERVER_ID; pkt.options[idx++]=4;

uint32_t sid = srv.sin_addr.s_addr;

memcpy(pkt.options+idx,&sid,4); idx+=4;

pkt.options[idx++]=OPT_END;

sockaddr_in dest{AF_INET,htons(CLIENT_PORT),INADDR_BROADCAST};

sendto(rawsock,&pkt,sizeof(pkt),0,(sockaddr*)&dest,sizeof(dest));

std::cout<<"Sent ACK\n";

}

}

}

📚 Explanation & Conclusion

- Client:

  - Sends DISCOVER broadcast.

  - Parses OFFER, then sends REQUEST for the offered IP.

  - Parses ACK and prints the leased IP.

- Server (minimal mock):

  - Listens on port 67.

  - On DISCOVER, responds with hardcoded OFFER (192.168.1.100).

  - On REQUEST, sends ACK.

✔ Summary

- Both use raw UDP sockets, ports 67/68.

- Packet structs (DhcpPacket) match the BOOTP/DHCP format.

- Options specify message type, server ID, requested IP.

- You can build real DHCP servers/clients by replacing hardcoded values, adding lease management and proper parsing.

Want help extending this into more robust code (subnet masks, lease tables, handling multiple clients)? Let